Table of Contents

# Earthquake Detection from Raw Waveforms via Relative Angle Matrix Image Encoding and Dual-Channel CNN–LSTM Networks

## Abstract

This report documents an investigation into applying the Relative Angle Matrix (RAM) method — a technique proposed by Wang and Zhao (2025, Applied Soft Computing 172) for converting one-dimensional vibration signals into two-dimensional images for convolutional neural network (CNN) classification — to seismic event detection from three-component (Z/N/E) waveform data. All comparisons are made against classic STA/LTA, the standard seismological trigger algorithm, rather than an arbitrary baseline.

The investigation proceeds in two phases. The first establishes a RAM-image CNN classifier, evaluates it against STA/LTA, and identifies a structural limitation: the RAM transform is provably scale-invariant, and therefore cannot represent absolute signal amplitude, which is the dominant feature short-window detection depends on. The second phase implements the source paper’s full dual-channel architecture — a CNN over the RAM image paired with an LSTM and multi-head self-attention branch over the raw waveform — corrects a design error identified by direct comparison against the source paper, introduces an amplitude auxiliary input to address the scale-invariance limitation, evaluates a spectrogram-based alternative to the RAM image, and compares three fusion strategies for combining the two channels.

The amplitude auxiliary input is the single largest contributor to classification performance measured in this work (test AUC 0.836 → 0.923 on the RAM-only classifier, an architecture-matched, single-variable comparison). Spectrogram-based encoding outperforms RAM-based encoding as the two-dimensional channel in every configuration tested. Gated fusion, a per-example alternative to the source paper’s fixed linear fusion, improves performance substantially on the spectrogram-based configuration but degrades it on both RAM-based configurations tested — a result reported in full rather than simplified, since a tidy explanation is not yet supported by the evidence.

Every numerical claim in this report is drawn directly from a measurement taken against the code in this repository. Where a result has not been verified computationally, or has not been repeated across random seeds, that limitation is stated explicitly rather than implied.

## 1. Introduction

Wang and Zhao (2025) propose the Relative Angle Matrix (RAM) method for converting one-dimensional bearing-vibration signals into two-dimensional images for CNN-based fault diagnosis. Seismic waveform data shares the same basic structure — a time series — and three-component (Z/N/E) seismic stations map naturally onto a three-channel image, making this a plausible transfer target. The central question addressed in this report is whether RAM encoding combined with a CNN, and subsequently with the source paper’s full dual-channel CNN–LSTM architecture, can separate a genuine seismic event from ambient noise directly from raw waveforms, and how such a system compares to the established STA/LTA trigger algorithm.

This work was conducted independently, outside assigned project tasks, using publicly available data (STEAD-format HDF5 chunks and self-downloaded FDSN MiniSEED archives), and therefore did not require navigating data-access or ethics constraints beyond standard public-data terms of use.

## 2. The RAM Transform, Formally Defined

### 2.1 Definition

Given a window of m samples v=[v1,…,vm] from one channel:

Step 1 — Standardization.

μ=1mi​vi,  σ=1mi​(vi−μ)2,  xi=vi−μσ

with σ←max(σ,ε), ε=10−12, to guard against flatlined channels.

Step 2 — Reshaping into local feature vectors. For a target image resolution n (--target-n), the column depth is derived as

d=max2, m/n

and x is zero-padded (or truncated) to length dn and reshaped column-wise into M∈Rd×n. Each column Xi∈Rd is a local feature vector — a contiguous d-sample segment of the window.

Step 3 — Angles relative to the centroid. With the central vector X=1ni=1nXi,

βi=arccosXi⋅XXi X∈[0,π]

The cosine argument is clipped to [−1,1] before the inverse cosine is taken; floating-point error can otherwise push it marginally outside this range, producing NaN — a failure mode not addressed in the source paper.

Step 4 — Pairwise angle differences.

Rij=βj−βi,  R∈Rn×n

Step 5 — Rendering to 8-bit. Since each β∈[0,π], it follows that Rij∈[−π,π] exactly, so a fixed affine map is used:

Pij=round255⋅clip(Rij,−π,π)+π2π

The fixed range is significant: a per-image min–max normalization (as used in the deprecated src/preprocessor/ path) rescales every window by its own extremes, which destroys comparability between windows. Under the fixed map, a given pixel value represents the same angle difference in every image in the dataset.

### 2.2 Structural Properties, Verified Numerically

The following are properties of the transform itself, confirmed to machine precision. They determine, independent of any model, what information a CNN operating on these images can possibly recover.

(a) The matrix is antisymmetric and rank-2. In matrix form R=1β⊤−β1⊤, hence

R⊤=−R,  Rii=0,  rank(R)≤2

Measured: max|R+R⊤|=0, the diagonal is exactly 0, and the singular value decomposition yields exactly two nonzero singular values of equal magnitude, with the remainder zero.

(b) The image carries n−1 degrees of freedom, not n2. Because R is generated by the vector β up to a global offset, an entire 64×64=4096-pixel channel is reconstructible from 63 numbers. This was verified by recovering β from the first column of R and reconstructing R in full; reconstruction error was exactly zero.

A 64×64×3 RAM image (12,288 pixels) therefore carries at most 189 independent values. The transform is a severe, lossy compression of the 6,000-sample (60 s) or 600-sample (6 s) input, and a CNN’s two-dimensional spatial processing operates on a highly redundant embedding of a one-dimensional signal.

(c) Every RAM image shares the same mean pixel value. Antisymmetry implies ij​Rij=0, so the mean pixel is exactly the map’s midpoint (127.5; measured 127.508 after integer rounding). The DC component of every image is constant by construction and carries no information.

(d) The transform is exactly scale-invariant, and shift-sensitive. For any c>0, scaling x→cx scales every Xi and X by c, and the cosine ratio cancels the factor exactly:

(cXi)⋅(cX)cXicX=c2(Xi⋅X)c2XiX

Measured: max|RAM(x)−RAM(37.5x)|=8.9×10−16, i.e. exact cancellation to floating-point precision. A mean shift, by contrast, does not cancel (max|ΔR|=1.31 rad for a representative offset). Section 8.2 develops the consequence of property (d), which is the most consequential finding of the first investigation phase.

### 2.3 Short Windows Are Structurally Disadvantaged

The reshape step ties image resolution to feature-vector length: with n fixed at 64, the segment length d=⌈m/n⌉ collapses as the window shortens (all figures at 100 Hz):

| Window | Samples m | d (samples/vector) | Segment duration | Zero-padding |
| --- | --- | --- | --- | --- |
| 60 s | 6000 | 94 | 0.94 s | 0.3 % |
| 10 s | 1000 | 16 | 0.16 s | 2.3 % |
| 6 s | 600 | 10 | 0.10 s | 6.2 % |
| 3 s | 300 | 5 | 0.05 s | 6.2 % |

At 3 s, each βi is the angle between two five-dimensional vectors. The cosine similarity of short random vectors has high variance, so the angles become dominated by sampling noise rather than signal structure, and the resulting image is correspondingly noisy. This is a geometric limitation of the encoding, independent of model capacity or dataset size, and it applies equally to classification and regression tasks.

The tension has no clean resolution within the current design: increasing d requires decreasing n (a smaller image), and at 3 s a target resolution of n=3 would be required to reach 60 s-equivalent segment lengths. Reducing --target-n was tested and produced results in the same accuracy range as other short-window variants (Section 7.2).

### 2.4 Three-Channel Composition

Each component (Z, N/1, E/2) is RAM-transformed independently and stacked into RGB channels: R = Z, G = N-like, B = E-like. Component selection is by role rather than alphabetical ordering, and stations lacking a usable vertical component are excluded (Section 6, defect 13).

One question was identified but not pursued in this phase: whether combining the three components before the transform (for example, by vector magnitude) would preserve inter-channel amplitude relationships better than transforming each independently. Given property 2.2(d), any such variant should be evaluated specifically for its effect on amplitude information.

## 3. Data Processing Pipeline

The pipeline comprises four stages, implemented in the seismic_cli package of the data_downloader repository (core.py, anchor.py, eval_baseline.py, cli.py).

### 3.1 Acquisition (src/download.py)

For each catalog event, stations within SEARCH_RADIUS_DEG = 0.5° (approximately 55 km) are resolved via FDSN (KOERI), with lookups cached on a roughly 1.1 km coordinate grid so co-located events share one metadata query. All windows for an event are fetched in a single bulk request and sliced in memory.

- Earthquake windows: 60 s from origin time.

- Noise windows: 300 s slices at −3 h and −6 h relative to origin.

- Contamination check: a noise window is discarded if any event in the unfiltered catalog falls within ±300 s of it. Checking against the filtered catalog would allow sub-threshold events to pass silently into the noise class; the buffer is deliberately wide because coda from larger events can persist for several minutes.

This check is purely temporal, so an event 500 km away will veto a candidate noise window. This is over-conservative and discards noise data that is otherwise scarce; adding a distance term is listed as a limitation in Section 9.

### 3.2 Arrival Anchoring (seismic-cli anchor-windows)

Short windows sliced from origin time can miss the P-wave arrival entirely at distant stations. At 6 s, an arrival later than 6 s after origin time means the nominal “earthquake” window contains no earthquake signal. Anchoring re-derives short windows from already-downloaded 60 s data without requiring re-download.

The pick uses the classic STA/LTA characteristic function

CF(k)=1nSTAj=k−nSTA+1kxj21nLTAj=k−nLTA+1kxj2

with the first trigger_onset crossing of trigger_on = 3.5 taken as the arrival sample a. The window is then cut as

[ a−f⋅T, a−f⋅T+T ),  f=pre_arrival_fraction=0.2

so that 20% of the window precedes the arrival (at 6 s: 1.2 s before, 4.8 s after).

Two correctness requirements were violated prior to this round of corrections (Section 6, defects 6–7): the trace must be detrended before the characteristic function is computed, and the pick should prefer the vertical component with fallback to horizontal components. A diagnostic block now reports, per run, the number of stations seen, skipped, picked on the vertical component, picked via fallback, and unpicked, together with how close failed picks came to the trigger threshold.

### 3.3 Dataset Generation (seismic-cli generate-dataset)

Per window, per channel, processing proceeds: linear and constant detrend → 5% Hann taper → fourth-order Butterworth bandpass (1–45 Hz, zero-phase) → RAM transform → 8-bit rendering → RGB stacking → PNG output.

Dataset generation enforces five constraints that required several iterations to implement correctly.

Station-disjoint splits, unified across classes. Each station is assigned to exactly one of train, validation, or test, and both its earthquake and noise windows follow that assignment. With approximately 97% of earthquake stations also contributing noise data, allocating the two classes independently — the earlier behavior — allowed nearly every station to appear in the training split under one label and the test split under the other (Section 6, defect 1).

Per-window station caps (--max-windows-per-station). Enforced by assigning each (station, file) pair an evenly spaced window quota rather than dropping entire files. Filenames retain the original window index w, so the sample range [ ws, ws+T) — with step s=T(1−overlap) — remains recoverable for baseline reconstruction even after subsampling.

Maximum-size balanced mode (--max). Assigns every usable station to the split with the largest relative deficit against ratio-proportional targets, then balances classes per split by trimming the surplus class via largest-remainder proportional rounding over per-file quotas. Without this mode, generation stops once global targets are filled and silently discards every remaining station, which is costly precisely where station diversity is already scarce.

Gap rejection. Traces are merged without interpolation fill, so gaps remain masked; gaps are then linearly filled for filtering purposes while a boolean mask records which samples are synthetic. Any window whose worst channel exceeds 5% synthetic samples is rejected. Previously, fill_value='interpolate' converted telemetry gaps into linear ramps that entered training as real signal.

Per-station sampling rates. Window sizing uses each station’s own sampling rate, recorded in the manifest, rather than assuming that the first trace’s rate in a file applies to every station represented in it.

Output is an ImageFolder directory tree together with manifest.csv (columns: split, class_name, station_key, file_path, filename, fs), one row per image, sufficient to reconstruct the exact source samples for any given image.

### 3.4 Baseline Standardization (--baseline)

Optionally, each channel may be standardized against that station’s long-term noise statistics (μsta,σsta) — accumulated in a streaming pass over all noise files for the given (station, component) pair, requiring at least 60 s of usable data — instead of the window’s own (μ,σ). The intent was to give the pipeline the long-term amplitude memory that constitutes STA/LTA’s principal advantage.

This does not achieve its intended effect, for the structural reason established in Section 8.2.

## 4. Model Architecture

ImprovedSeismicCNN (src/model/cnn_train.py) is a ResNet-style CNN with Squeeze-and-Excitation blocks and a single-logit binary output.

### 4.1 Components

Residual block. For input u:

ResBlock(u)=GELUSEBN2(W2*GELU(BN1(W1*u)))+S(u)

using 3×3 convolutions, the first of which carries the stride-2 downsample, and S the identity mapping or a 1×1 projection when the input and output shapes differ.

Squeeze-and-Excitation (reduction r=16), applied before the residual addition. Given z∈RH×W×C:

s=GAP(z)∈RC,  e=σWb ReLU(Was)∈(0,1)C,  SE(z)c=ec⋅zc

that is, a learned per-channel gain conditioned on global context. Because the three input channels correspond to Z/N/E components, channel attention has a direct physical interpretation: it can learn to weight components differently, and vertical components typically carry the cleanest P-wave onset.

Classification head. Global average pooling → Dropout(p_1) → Linear(C_f, h) → GELU → Dropout(p_2) → Linear(h, 1), producing a single logit.

### 4.2 Dimensions and Parameter Budget (64×64×3 Input, Measured)

| Stage | Output (C,H,W) | Parameters |
| --- | --- | --- |
| in_conv (3→16) | (16, 64, 64) | 448 |
| layer1 (16→32, stride 2) | (32, 32, 32) | 14,656 |
| layer2 (32→64, stride 2) | (64, 16, 16) | 58,240 |
| layer3 (64→128, stride 2) | (128, 8, 8) | 232,192 |
| layer4 (128→256, stride 2) | (256, 4, 4) | 927,232 |
| GAP → head (256→64→1) | (1,) | 16,513 |
| Total (long preset) |  | 1,249,297 |
| Total (short preset, 3 stages, head width 32) |  | 309,713 |

layer4 alone accounts for 74% of the long-preset parameter count, which is why the short preset omits it, reducing the model by a factor of 4.0.

### 4.3 Training Procedure

The loss function is BCEWithLogitsLoss applied to label-smoothed targets y=0.8y+0.1, which bounds the confidence the model is rewarded for. This introduces a floor on training loss: at the optimum, per-sample binary cross-entropy equals the binary entropy H(0.1)≈0.325 nats, so smoothed training loss is not directly comparable in magnitude to unsmoothed validation loss. An unsmoothed training-loss diagnostic is therefore logged alongside the smoothed value, so the train/validation gap can be interpreted correctly (Section 6, defect 9).

The optimizer is AdamW with gradient-norm clipping at 1.0 and mixed precision on CUDA. Checkpointing retains the best epoch by the monitored metric, and final test evaluation always loads that checkpoint.

### 4.4 Window-Length Presets

Short-window training runs overfit the full network within approximately 10 epochs, while 60 s runs do not. --window-seconds therefore selects a preset (12 s or shorter selects short); any explicitly passed flag overrides its preset value, and omitting the flag reproduces the original configuration exactly.

|  | long (60 s) | short (≤ 12 s) | Rationale |
| --- | --- | --- | --- |
| Stages / head width | 4 / 64 | 3 / 32 | Reduces parameters from 1.25 M to 0.31 M |
| Dropout p1,p2 | 0.5, 0.3 | 0.6, 0.4 | Stronger regularization on the head |
| Weight decay | 1e-2 | 3e-2 | Stronger regularization |
| RandomErasing | Off | p=0.25 | The only label-safe augmentation available; flips or rotations would scramble the RAM matrix’s temporal ordering, since axis position encodes time |
| Batch size / learning rate | 128 / 1e-4 | 64 / 2e-4 | More updates per epoch on a smaller dataset |
| Schedule | ReduceLROnPlateau | Cosine annealing | Plateau scheduling reacts too slowly when the performance peak arrives by roughly epoch 6 |
| Checkpoint metric | Validation loss | Validation AUC | Validation cross-entropy degrades from calibration drift while ranking quality continues to improve; loss-based selection risks saving a pre-peak model |

## 5. Baseline Method: STA/LTA on Identical Windows

The baseline scores the same characteristic function as Section 3.2, taking maxkCF(k) over each channel and the maximum across channels as the window’s score. Two properties make the comparison fair.

- Exact-window reconstruction. Windows are rebuilt from the raw MiniSEED data via the manifest (same file, station, and window index), not resampled independently.

- Window-adaptive parameters. Fixed parameters (sta=1.0, lta=10.0) cannot be computed inside a 3 s or 6 s window. Parameters are instead derived as LTA=min(10, T/3) and STA=max(0.05, LTA/10), which reproduces the classic 1/10 ratio exactly at 60 s and gives 0.2/2.0 at 6 s and 0.1/1.0 at 3 s.

Reported metrics are AUC — threshold-free, the appropriate comparison against the CNN — together with accuracy, precision, and recall at the Youden’s-J threshold. That threshold is selected on the evaluated split, making it an oracle: the thresholded figures represent STA/LTA’s upper bound rather than a like-for-like comparison against the CNN’s fixed 0.5 cutoff.

## 6. Software Defects Identified and Corrected

The reliability of the results in this report depends substantially on defects identified and corrected during development. Defects 1–5 predate this round of work; defects 6–13 were identified during a systematic audit of the full repository.

| # | Defect | Mechanism | Impact |
| --- | --- | --- | --- |
| 1 | Cross-class station leakage | Splits were allocated independently per class; with approximately 97% station overlap, a station could appear as train-earthquake and test-noise | The model could exploit station identity as a shortcut; biased measured test performance downward, most severely where station counts are low (short windows) |
| 2 | Station caps were ineffective | The cap was applied at file granularity; a single 300 s noise file (roughly 200 windows at 3 s) exceeded any smaller cap and was retained in full | Explains observations of only 2–4 distinct noise stations in validation/test even after capping |
| 3 | Fixed-resolution collapse | Reshape depth d was fixed regardless of window length | 3 s windows collapsed to approximately 3×3 images; corrected by deriving d from the target resolution |
| 4 | Origin-anchored short windows | Short windows were cut from origin time rather than arrival time | A meaningful fraction of nominal “earthquake” windows contained no signal |
| 5 | Noise/earthquake station mismatch | Noise data was sourced independently of earthquake stations (approximately 47% overlap) | The model rarely observed both classes from the same instrument; a targeted downloader raised overlap to approximately 97% |
| 6 | STA/LTA computed on raw counts (anchoring) | classic_sta_lta was computed on un-detrended MiniSEED counts; a large DC offset pins the characteristic function near 1 | Verified: a synthetic arrival with a 106-count offset produces a maximum characteristic-function value of exactly 1.000 (no pick possible on any channel); after detrending, the pick lands within 4 samples of ground truth |
| 7 | Arrival pick on a horizontal component | sorted(traces)[0] selects the E component before Z alphabetically, with a single attempt and no fallback | P-wave onsets are cleanest on the vertical component; corrected to vertical-first with fallback through remaining channels |
| 8 | STA/LTA computed on raw counts (baseline evaluation) | The same DC-offset defect present in the baseline scorer | Baseline performance was understated at high-offset stations, plausibly explaining a measured STA/LTA AUC swing of 0.78–0.98 between data pulls |
| 9 | Label-smoothing asymmetry | Smoothing was applied to training targets only; smoothed binary cross-entropy floors near 0.325 nats | Training and validation loss curves were not directly comparable in magnitude; validation metrics were unaffected, since they always used unsmoothed labels |
| 10 | Threshold mismatch | cnn_from_tensor.py validated at a 0.60 threshold but tested at 0.50 | Validation and test accuracy were measuring different decision rules |
| 11 | Gap interpolation treated as signal | merge(fill_value='interpolate') fabricates linear ramps across telemetry gaps | Synthetic data was trained on as though genuine; corrected via gap masking, with windows exceeding 5% synthetic samples rejected |
| 12 | Single-rate assumption | The first trace’s sampling rate in a file was applied to every station represented in it | Produced an incorrect physical window duration for off-rate stations; corrected to use per-station sampling rate |
| 13 | Alphabetical channel selection | sorted(keys)[:3] could select ['1','2','E'] — two horizontal components and no vertical | Silent component mis-assignment; corrected to role-based selection requiring a vertical component |

Two items were suspected as defects but confirmed not to be: the RAM mathematics as implemented transcribes the source paper correctly, including guards the paper itself omits (the ε floor on σ, and clipping before the inverse cosine); and the manifest’s window-index-to-sample mapping is exact.

## 7. Initial Results (Pre-Correction Baseline)

All figures in this section predate the Section 6 corrections to defects 6–13 and are reported for continuity rather than as validated results. In particular, every STA/LTA figure was produced by the DC-offset-affected scorer, and every short-window figure by the ineffective station cap and the cross-class split allocation defect. These comparisons require re-running end-to-end before any figure in this section should be treated as current.

### 7.1 60-Second Windows

| Metric | CNN | STA/LTA |
| --- | --- | --- |
| Accuracy | 89.61 % | — |
| Recall (earthquake) | 0.963 | — |
| Precision (earthquake) | 0.817 | — |
| ROC-AUC | — | 0.7777 |

Test set: 2,561 samples, station-disjoint. Across repeated dataset regenerations, the CNN’s margin over STA/LTA on AUC held in the range of approximately 0.10–0.15 AUC points and, if anything, widened across data pulls. This is the headline result of the initial phase, and the one most likely to survive re-validation, though the baseline figure itself is expected to change once defect 8 is corrected, so the margin must be re-measured rather than assumed to hold.

### 7.2 6-Second Windows

Most recent pre-correction run (853 test samples):

Accuracy 72.80 %
              precision  recall  f1-score  support
   noise         0.7203  0.7727    0.7456      440
   earthquake    0.7375  0.6804    0.7078      413
Confusion: TN=340  FP=100  FN=132  TP=281

Three pipeline-side variants — per-window standardization, per-station baseline standardization (74.88% versus 77.99% at the time, i.e. no measurable improvement), and a smaller --target-n — all produced results in a 72–78% band, with the same overfitting signature: validation AUC peaking around epoch 4–10, followed by flattening or degradation while training loss continued to decrease.

The convergence of three unrelated interventions on the same performance band motivated both the model-side audit summarized in Section 4.4 and the structural analysis in Section 8. At approximately 850 samples, binomial noise on accuracy is roughly ±1.5 percentage points; differences under approximately 3 percentage points between runs should not be interpreted as meaningful.

## 8. Analysis

### 8.1 Three Compounding Causes of Short-Window Underperformance

- Geometric (Section 2.3). At 6 s, local feature vectors are 10 samples long; at 3 s, 5 samples. Cosine angles between very short vectors are noise-dominated, so the encoding degrades before any model observes it.

- Statistical. Short-window datasets were small (approximately 1.5–4 k samples) and, prior to the station-cap correction, drawn from very few distinct noise stations. The 1.25 M-parameter network is heavily over-provisioned at this scale (roughly 300–800 parameters per sample), consistent with the observed early-peak overfitting curve.

- Informational (Section 8.2). The single feature that most cleanly separates a 6 s earthquake window from noise — amplitude relative to the station’s background level — is provably absent from the model’s input.

### 8.2 The RAM Transform Discards Amplitude, and Amplitude Is What STA/LTA Uses

This is the most consequential finding of the first investigation phase.

By property 2.2(d), the transform is exactly scale-invariant: RAM(cx)=RAM(x). Consider what baseline standardization actually changes. After clean_and_filter_1d (detrend, demean, bandpass), the window mean is approximately 0, and μsta is likewise approximately 0, since it is accumulated from identically cleaned noise data. The two standardization modes therefore differ, to a close approximation, by a single factor σwin/σsta — a pure rescaling, which RAM cancels exactly.

Measured on representative windows, comparing self-standardized and baseline-standardized images at the same station:

| Event strength | σwin/σnoise | Mean pixel difference | Maximum |
| --- | --- | --- | --- |
| Weak (SNR ≈ 2) | 1.84 | 0.63 / 255 levels | 3 |
| Strong (SNR ≈ 20) | 19.19 | 0.27 / 255 levels | 1 |

The differences are sub-level, and, decisively, do not grow with the amplitude ratio: a 20-times stronger event produces a slightly smaller image difference, not a larger one. The residual difference originates from the near-zero post-filter mean (RAM is shift-sensitive, property 2.2(d)), not from amplitude.

Three conclusions follow.

- The --baseline flag cannot deliver its intended effect. The amplitude and SNR information it is designed to preserve is eliminated by the very next step in the pipeline. The earlier 6 s result (74.88% versus 77.99%) was not evidence against the long-term-memory hypothesis; it was run-to-run noise on near-identical images. The hypothesis stated in an earlier version of this report was never actually tested.

- This explains STA/LTA’s competitiveness at short windows. Its entire discriminative signal is amplitude measured against a long-term baseline — precisely the quantity the RAM pipeline structurally cannot represent. The CNN is being asked to win using shape and frequency structure alone.

- This predicts that magnitude regression on RAM images will underperform. Local magnitude is essentially log peak amplitude with a distance correction; a scale-invariant encoding removes the dependent variable’s principal predictor. Retrying regression with --baseline alone would not address this limitation.

### 8.3 Implication: Amplitude Belongs as an Explicit Input

The indicated correction is not a further image variant, but an auxiliary scalar input. For each window and channel, compute

SNRlog=logσwinσsta,noise

store it in the manifest, and concatenate it — together with epicentral distance, needed to disambiguate magnitude from distance — to the pooled CNN features prior to the classification head. This restores the discarded quantity without altering the transform itself.

A three-step validation order was proposed before committing further CNN training runs to this hypothesis:

- Confirm the near-no-op directly: generate a sample of windows with and without --baseline and difference the resulting images (expected result: at most 3 levels, per the table above).

- Regress magnitude on log(σwin/σnoise) and log(distance) alone, using a linear model or gradient- boosted tree. This is effectively fitting a local-magnitude relation; if it fails, the station baselines are too noisy for a CNN to inherit anything useful from them.

- Only then train the joint (image and scalar) model — both for regression and for the 6 s classifier, where this input hands the network the exact feature STA/LTA has been outperforming it with.

Section 10 reports the outcome of this validation order.

## 9. Limitations and Recommendations (First Investigation Phase)

Immediate, blocking re-validation:

- Regenerate all datasets and re-run every comparison under the Section 6 corrections, particularly the 60 s CNN-versus-STA/LTA margin, whose baseline term is expected to change.

- Re-run 6 s generation with --max and a functioning per-window cap; station diversity should improve substantially, since the previous mode discarded every station beyond the bottleneck class target.

- Re-run 6 s training under the short preset, then run ablations (--num-stages 4 isolates the effect of regularization; --monitor loss isolates the effect of capacity) across at least three seeds before treating any single result as conclusive.

Structural:

- Add the amplitude and distance auxiliary input described in Section 8.3 — the highest-expected-value change identified, and a prerequisite for revisiting magnitude regression.

- Given property 2.2(b) — 4,096 pixels carrying 63 degrees of freedom — evaluate whether feeding the β vector directly to a one-dimensional model matches the two-dimensional CNN’s performance. If it does, the image encoding contributes computational cost without additional information, which would be an important finding in its own right.

- Compare combine-then-RAM (vector magnitude across components) against per-channel RAM.

- Add a distance term to the noise-contamination check; the current time-only rule discards usable noise data that is otherwise scarce.

Standing caveats:

- This investigation has not undergone external peer review.

- The 60 s results are the strongest obtained but predate the Section 6 corrections; the 6 s results are preliminary.

- Short-window noise-station diversity remains limited, so precision and recall figures at short windows should not be interpreted beyond one significant figure.

- All conclusions in Section 8.2 concern this transform as specified; they follow from scale invariance and would not apply to an amplitude-preserving variant.

## 10. Extension: The Full Dual-Channel Architecture and the Amplitude Fix

Sections 1–9 address RAM encoding paired with a CNN alone — one of the two channels described in the source paper. This section adds the second channel (LSTM with multi-head self-attention over the raw signal) to evaluate the complete architecture, and then implements the correction proposed in Section 8.3. All results in this section are fresh runs on the corrected pipeline (post Section 6, defects 1–13); they do not carry the pre-correction caveat attached to Section 7.

All results below share a common dataset: seismic-cli generate-*-dataset --max on 6 s arrival-anchored windows, 71,672 windows total (35,836 per class, balanced), station-disjoint (82/30/40 earthquake stations and 104/35/38 noise stations across train/validation/test). Code is located in data_downloader/seismic_cli/ (ram_dual.py, ram_aux.py, spectrogram.py) and cnn_earthquake/src/ (cnn_lstm.py, cnn_lstm_classify.py, cnn_lstm_classify_aux.py, cnn_lstm_stack.py, cnn_ram_aux.py). As in Section 7.2, each configuration reflects a single train/validation/test split rather than a repeated measurement across seeds; differences under approximately 1–2 points should be treated as noise rather than as an established effect.

### 10.1 The Source Paper’s Architecture (1D2D-EDL)

Two independent channels operate on the same window and are fused:

1D: F1D=MSALSTM(x)  2D: F2D=CNNRAM(x)  F′=a F1D+b F2D

with a,b learned scalars. This is implemented as LSTMAttentionBranch (bidirectional LSTM, followed by nn.MultiheadAttention with a residual connection and layer normalization, matching a standard transformer block, followed by mean-pooling over time) and CNNBranch (three convolution/ batch-norm/GELU stages with global average pooling, which is resolution- agnostic and therefore accepts either a square RAM image or a non-square spectrogram without modification). Both components reside in cnn_earthquake/src/cnn_lstm.py, shared with the unrelated catalog-forecasting model implemented in the same file.

A design error, identified and corrected by consulting the source paper directly rather than a prior summary of it. The initial implementation of the 1D branch supplied the LSTM with the (n,d) chunk matrix from which the RAM image’s angle vector β is computed, on the premise that both channels should observe the same reshaped data. This does not match the source paper. Section 3.3.1 and Figure 7 of Wang and Zhao (2025) state explicitly that the one-dimensional time series is normalized and then input directly into the LSTM model; the RAM reshape feeds only the CNN channel. The two channels are independent feature extractors over the same raw signal, not two views of a shared intermediate representation. The function supplying the incorrect design (core.ram_matrix_and_chunks()) was subsequently removed; core.ram_matrix() was verified to produce output byte-identical to its pre-refactor version on both occasions this change was made.

One consequence of the correction carries a computational cost: multi-head self-attention over the full raw window is O(m2). At 100 Hz, a 6 s window is m=600 (an attention matrix of 360,000 entries — trivial); a 60 s window is m=6000 (36 million entries per attention head — substantially heavier, and likely to require a smaller batch size). This is an inherent cost of matching the source paper’s design, not a defect.

### 10.2 Data Pipeline Additions

| Encoder | Output | Description |
| --- | --- | --- |
| RamDualEncoder | {seq, img} | seq is the raw standardized (m,3) Z/N/E waveform; img is the Section 2 RAM image |
| SpectrogramDualEncoder | {seq, img} | Same seq; img is a log-power spectrogram, produced by wrapping SpectrogramEncoder via composition so the two implementations remain identical wherever they overlap |
| RamAuxEncoder | {img, aux} | img is the RAM image; aux is [logSNR,logRMS] (defined below); no LSTM branch is present |
| RamDualAuxEncoder | {seq, img, aux} | All three components: RamDualEncoder plus the auxiliary vector |

logSNR=logσwin,cσsta,cc∈{Z,N,E}  logRMS=logσwin,cc∈{Z,N,E}

identical to the log_snr definition used in regression.py. The term σsta,c is obtained from compute_station_noise_baselines() (the mechanism described in Section 3.4), computed unconditionally in this context — independent of the --baseline flag, which controls only whether seq and img themselves use the station baseline or per-window self-standardization. This independence is deliberate: Section 8.2 establishes that RAM’s image content does not depend on which (μ,σ) pair is used for standardization, so gating log_snr behind --baseline would have made the correction unavailable by default with no offsetting benefit. Any window whose station lacks at least 60 s of usable noise data falls back to logSNR=0.

All four encoders resample every window to a nominal rate before standardizing (_resample_to/_fit_length), the same correction SpectrogramEncoder required for the image alone (Section 3.3’s per-station-rate constraint). In this context it additionally keeps every seq tensor’s sample count fixed regardless of a station’s native sampling rate, a constraint the image’s fixed target_n did not require.

### 10.3 Model Architecture Additions

- DualChannelBinaryNet implements the source paper’s architecture directly: LSTMAttentionBranch combined with CNNBranch, fused as aF1D+bF2D, with a single-logit head. The --channels argument (all, 1d, 2d) ablates either branch.

- RamAuxCNN uses the same ResNet-with-Squeeze-and-Excitation trunk as ImprovedSeismicCNN (Section 4.1, reusing ResBlock directly), with pooled features concatenated with the auxiliary vector before the classification head. The --no-aux flag removes the concatenation only, providing an architecture-matched control that isolates precisely what the two auxiliary scalars contribute.

- DualChannelAuxBinaryNet extends DualChannelBinaryNet with an auxiliary branch concatenated after the aF1D+bF2D fusion step, following the same pattern already used by the unrelated catalog model’s DualChannelRiskNet. The --channels argument extends to {all, 1d, 2d, aux, 1d+aux, 2d+aux}.

- cnn_lstm_stack.py implements late-fusion stacking: given two already-trained, frozen checkpoints (for example, --channels 1d and --channels 2d runs), it collects their pre-sigmoid logits and fits sklearn.LogisticRegression — one weight per branch plus a bias, in logit space — on validation-set logits only, which were not observed by either frozen model’s own training, and evaluates on the test set. No backbone retraining is involved.

- GatedFusion (cnn_lstm.py, shared by both classification scripts) replaces the fixed scalar pair with a per-example gate: g=σ(MLP([F1D,F2D])), followed by gF1D+(1−g)F2D. Selected via --fusion {linear, gate} in both cnn_lstm_classify.py and cnn_lstm_classify_aux.py; the option affects only channel combinations in which both branches are active. Gate-value diagnostics (mean value by true class, and by prediction correctness) are reported at test time in place of the linear mode’s learned-weight summary.

### 10.4 Results

Every row reflects 6 s windows on the same 71,672-window dataset, with a test set of 9,548 samples (balanced). Parameter counts are measured directly from each trained model, not estimated.

| Model | Parameters | Test AUC | MCC | Accuracy |
| --- | --- | --- | --- | --- |
| Spectrogram CNN only (2d) | 115,459 | 0.9793 | 0.8666 | 93.28 % |
| Spectrogram-dual, fused, linear (all) | 182,563 | 0.9646 | 0.8122 | 90.61 % |
| RAM-dual + aux, fused, linear (all) | 182,759 | 0.9514 | 0.7790 | 88.95 % |
| RAM-dual + aux, no LSTM branch (2d+aux) | 115,655 | 0.9468 | 0.7775 | 88.84 % |
| RAM + aux, no dual architecture (use_aux=True) | 309,777 | 0.9230 | 0.7018 | 84.79 % |
| RAM-dual, raw waveform only (1d) | 76,707 | 0.9216 | 0.6849 | 84.22 % |
| RAM-dual, fused, linear, no aux (all) | 182,563 | 0.9144 | 0.6042 | 79.57 % |
| RAM CNN only, no aux (use_aux=False) | 309,713 | 0.8356 | 0.5339 | 76.70 % |
| RAM-dual, RAM image only (2d) | 115,459 | 0.8408 | 0.5288 | 76.42 % |

Late-fusion stacking (Section 10.3), fit on frozen 1d/2d logits:

| Base checkpoints | 1d alone | 2d alone | Naive average (logits) | Stacked |
| --- | --- | --- | --- | --- |
| RAM-dual | AUC 0.9229, MCC 0.688 | AUC 0.8408, MCC 0.530 | AUC 0.9136, MCC 0.692 | AUC 0.9203, MCC 0.688, accuracy 84.31 % |
| Spectrogram-dual | AUC 0.9229, MCC 0.688 | AUC 0.9793, MCC 0.867 | AUC 0.9697, MCC 0.866 | AUC 0.9743, MCC 0.871, accuracy 93.54 % |

Learned linear-fusion weights (a,b), for reference: RAM-dual without aux, (0.996,0.829); spectrogram-dual, (0.749,1.051); RAM-dual with aux, (0.499,0.354). The last pair is markedly more balanced in relative terms once both branches carry usable information, rather than one weight sitting near 1 with the other trailing.

### 10.5 Analysis

10.5.1 The source paper’s fixed linear fusion underperformed the best single branch, on two independent representations, before the amplitude correction. With RAM as the 2D channel, the raw-waveform branch alone (AUC 0.9216) outperformed the fused model (0.9144). With a spectrogram as the 2D channel, the image branch alone (0.9793) outperformed the fused model (0.9646). A fixed pair of scalars, trained jointly with both branches, cannot suppress a weaker branch on the specific examples where it is incorrect, and joint training can allow a noisy branch to degrade the stronger branch’s own learned representation. This pattern reproduced on two independent 2D representations, which is why it is treated as a property of the fusion mechanism rather than of either individual branch.

10.5.2 Late-fusion stacking recovers what joint fusion lost, without modifying either branch. Freezing the same two checkpoints and fitting a two-input logistic regression on their logits (Section 10.3) matched or improved on the joint-fusion result in both cases tested. On RAM-dual, the stacked result lands within measurement noise of the best single branch (0.9203 versus the 1d-alone figure of 0.9229) rather than below it. On spectrogram-dual, stacking outperforms both single branches outright (0.9743 AUC, 0.871 MCC, versus 2d-alone’s 0.9793 AUC and 0.867 MCC); stacking’s AUC is marginally below the 2d-alone figure even though its MCC and accuracy are higher, indicating a small trade of ranking quality for a better decision at the fixed 0.5 threshold, a difference within this dataset’s established noise floor. This result confirms that the fusion problem identified in 10.5.1 originates in the joint-training mechanism rather than in an inherent limitation of combining the two branches.

10.5.3 The amplitude auxiliary input is the largest single effect measured in this investigation, and confirms the prediction made in Sections 8.2 and 8.3 quantitatively. In an architecture-matched, single-variable comparison (identical ImprovedSeismicCNN-derived trunk, differing only in whether the auxiliary concatenation is present):

AUC: 0.8356→0.9230 (+0.0874)  MCC: 0.5339→0.7018 (+0.1679)  Accuracy: 76.70%→84.79% (+8.09 pp)

Two scalars, computed at negligible cost from data the pipeline already possessed, closed most of the gap between a plain RAM classifier and STA/LTA-competitive performance. This correction also incidentally resolved the fusion pathology identified in 10.5.1: the amplitude-augmented dual model (all, AUC 0.9514) outperforms every single-branch RAM alternative, which the equivalent model without the auxiliary input failed to do. The 2d+aux ablation (no LSTM branch present, AUC 0.9468) lands almost exactly on the full fused model’s result, indicating that the amplitude correction accounts for nearly all of the improvement, and that the LSTM branch contributes only a small increment beyond it, rather than being structurally necessary to compensate for what the RAM image cannot represent.

10.5.4 Spectrograms remain the stronger 2D representation, with or without the amplitude correction. Spectrogram-2D-only (0.9793 AUC) outperforms RAM-plus-amplitude-2D-only (0.9468 AUC) by a substantial margin. This is expected and does not contradict 10.5.3: --normalize station spectrograms preserve amplitude as a function of frequency and time, a considerably richer representation than two collapsed scalars appended after the fact. The amplitude correction makes RAM competitive with the raw-waveform branch and with its own pre-correction performance; it does not make RAM’s 2D representation as informative as a spectrogram’s. Where RAM images are specifically required — for example, for compatibility with the source paper’s method — pairing them with the amplitude auxiliary input is a substantial, low-cost improvement. Where the 2D representation is an open choice, spectrograms remain the stronger option on this dataset.

10.5.5 Gated fusion produces mixed results across three tested configurations, and does not support a single unifying explanation on the evidence available. GatedFusion was evaluated against linear fusion on three configurations: spectrogram-dual, RAM-dual without the amplitude input, and RAM-dual with it.

| Configuration | Linear fusion | Gated fusion | Best single branch | Mean gate g |
| --- | --- | --- | --- | --- |
| Spectrogram-dual | AUC 0.9646, MCC 0.812, 90.61 % | AUC 0.9761, MCC 0.850, 92.51 % | 2d-alone: 0.9793 / 0.867 / 93.28 % | 0.169 |
| RAM-dual, no aux | AUC 0.9144, MCC 0.604, 79.57 % | AUC 0.9071, MCC 0.637, 81.68 % | 1d-alone: 0.9216 / 0.685 / 84.22 % | 0.719 |
| RAM-dual + aux | AUC 0.9514, MCC 0.779, 88.95 % | AUC 0.9487, MCC 0.744, 87.12 % | 2d+aux-alone: 0.9468 / 0.778 / 88.84 % | 0.487 |

The outcome differs by metric as well as by configuration. On spectrogram-dual, gated fusion improves all three metrics over linear fusion, closing most of the gap to the best single branch without the separate frozen-checkpoint procedure stacking requires. On RAM-dual without the amplitude input, gated fusion is mixed: AUC decreases (0.9144 → 0.9071) while MCC and accuracy both increase (0.604 → 0.637; 79.57% → 81.68%), indicating a shift in decision calibration at the fixed 0.5 threshold rather than a uniform change in ranking quality. On RAM-dual with the amplitude input, gated fusion is uniformly worse across all three metrics.

An initial hypothesis, formed after the first two configurations were measured, proposed that gating helps when branches are genuinely complementary and disagree meaningfully on a per-example basis, and can hurt when one branch already dominates. The third configuration does not cleanly support this account: the performance gap between branches on RAM-dual without aux (0.9216 versus 0.8408, a difference of 0.081) is larger than on spectrogram-dual (0.9793 versus 0.9216, a difference of 0.058), yet gating still underperformed linear fusion in the RAM-dual case. The more accurate statement, on the evidence collected, is that gated fusion improved results in exactly one of three tested configurations — the spectrogram-based one — and modestly degraded results in the two RAM-based configurations. Whether this reflects a property of the RAM representation specifically, an estimation difficulty in learning the gate function reliably from a comparatively noisier branch pair, or an artifact of single-seed measurement, is not established by the data available and should not be asserted. Gated fusion is not a default improvement over linear fusion, and its benefit or cost should be measured directly for any new branch pairing rather than assumed from this result.

10.5.6 A hyperparameter sweep of the RAM-plus-amplitude classifier (cnn_ram_aux.py) produced no distinguishable improvement over the established defaults. Six configurations were trained: the established default (learning rate 2e-4, weight decay 3e-2, dropout 0.6/0.4, head width 32, three residual stages), two alternative learning rates (1e-4, 3e-4), a lower weight decay (1e-2), a wider head alone (width 64, three stages), and the original “long” preset’s full capacity (four stages, dropout 0.5/0.3, head width 64 — approximately four times the parameter count). Selection was made on validation AUC, decided before training began, specifically to avoid selecting a configuration by its test-set performance across multiple candidates.

| Configuration | Parameters | Validation AUC | Test AUC | Test MCC | Test accuracy |
| --- | --- | --- | --- | --- | --- |
| Default | 309,777 | 0.9302 | 0.9230 | 0.7018 | 84.79 % |
| Learning rate 3e-4 | 309,777 | 0.9307 | 0.9268 | 0.7137 | 85.28 % |
| Weight decay 1e-2 | 309,777 | 0.9301 | 0.9228 | 0.7013 | 84.77 % |
| Learning rate 1e-4 | 309,777 | 0.9287 | 0.9224 | 0.7040 | 84.96 % |
| Four stages, wider head (4× capacity) | 1,249,425 | 0.9287 | 0.9302 | 0.7235 | 86.13 % |
| Wider head only (three stages) | 314,001 | 0.9268 | 0.9270 | 0.7054 | 85.17 % |

All six configurations fall within a validation AUC band of 0.9268–0.9307 — a spread of 0.0039, well inside the approximately 1–2 point noise floor already established for single-seed measurements on this dataset (Section 9). The nominal winner by the pre-specified selection rule (learning rate 3e-4, val AUC 0.9307) exceeds the default by 0.0005, which is not distinguishable from noise and should not be reported as a genuine improvement. One result is worth noting for methodological reasons rather than performance: the four-times-larger configuration scored lower on validation AUC (0.9287) than the 310,000-parameter default despite scoring higher on test AUC (0.9302) — a direct illustration of why selection was fixed to validation performance before training began, and further evidence that this task is limited by available information rather than by model capacity, consistent with the diagnosis in Section 8.1. On the evidence gathered, hyperparameter tuning is not a productive direction for this model; the established default configuration should be retained.

10.5.7 The amplitude auxiliary input, extended to the spectrogram-based model, does not repeat its RAM-side effect — and mildly hurts the 2D branch in isolation. SpectrogramDualAuxEncoder (Section 10.2) pairs the spectrogram-dual dataset with the same [log_snr, log_rms] vector used for RAM. Results, alongside the existing no-aux spectrogram figures for comparison:

| Configuration | Test AUC | MCC | Accuracy |
| --- | --- | --- | --- |
| Spectrogram 2D only, no aux | 0.9793 | 0.8666 | 93.28 % |
| Spectrogram + aux, no LSTM branch (2d+aux) | 0.9749 | 0.8626 | 93.02 % |
| Spectrogram-dual, no aux, gated fusion | 0.9761 | 0.8501 | 92.51 % |
| Spectrogram-dual + aux, fused, linear (all) | 0.9733 | 0.8465 | 92.31 % |
| Spectrogram-dual, no aux, fused, linear | 0.9646 | 0.8122 | 90.61 % |

Two results follow directly. First, adding the auxiliary input to the 2D branch alone makes it worse, not better: 2d+aux (0.9749) scores below plain 2d (0.9793), a decrease of 0.0044. This is the reverse of the RAM result (Section 10.5.3, an increase of 0.0874) and is consistent with the explanation offered in Section 10.5.4: a station-normalized spectrogram already encodes amplitude as a function of time and frequency, so appending two collapsed, redundant scalars adds estimation noise without adding information the model did not already have access to. Where Section 10.5.3 demonstrated that the auxiliary input matters when the 2D representation is scale-invariant, this result demonstrates the converse: it does not matter, and can mildly hurt, when the 2D representation already carries the same information in richer form.

Second, the auxiliary input still improves the fused model relative to its own no-aux baseline (0.9646 → 0.9733, +0.0087), for a reason similar to why gated fusion helped in the same setting (Section 10.5.5): both interventions reduce how much a naive linear combination is dragged down by a comparatively weaker branch, whether by learning to trust the stronger branch more or by handing the weaker branch’s fusion partner information it lacked. Neither intervention, alone or combined, reaches the ceiling established by 2d alone.

Considered together with Sections 10.5.1–10.5.6, the single best-performing configuration measured in this entire investigation remains the plain spectrogram CNN classifier, with no LSTM branch, no auxiliary input, and no fusion mechanism (AUC 0.9793). Every structural addition tested — the source paper’s dual-channel architecture, the amplitude auxiliary input, gated fusion, late-fusion stacking — improved on some other configuration along the way, and each produced a genuine, informative finding about why RAM underperforms and how fusion mechanisms behave. None of them, individually or in combination, has yet exceeded the simplest model in this comparison. This should be read as a genuine result rather than a failure of the investigation: it indicates that, for this task and dataset, the highest-value remaining work is more likely to be in feature representation (for example, per-component auxiliary inputs, or spectrogram parameters) than in additional architectural complexity layered on top of an already-strong 2D representation.

### 10.6 Updated Limitations and Recommendations

- Every result in Sections 10.4 and 10.5.5 reflects a single train/validation/test split at one random seed. None of the differences below approximately 1–2 points — including stacked-RAM-dual versus 1d-alone, 2d+aux versus the full amplitude-dual model, and gated versus linear fusion on either RAM-dual configuration — should be treated as more than suggestive without repeated measurements across seeds, the same caution already applied to Section 7’s figures in Section 9.

- The amplitude correction has now been evaluated on the spectrogram-dual model (Section 10.5.7) but has not been combined with stacking (amplitude-augmented branches, frozen, then stacked). This is a natural next experiment and is inexpensive given that the relevant datasets and checkpoints already exist.

- Gated fusion has not been combined with stacking (for example, a gate applied to frozen rather than jointly trained branch features), nor with the amplitude auxiliary input on the spectrogram-dual model specifically (Section 10.5.7 used linear fusion only). Given that gating already matched most of what stacking achieved on spectrogram-dual without aux, whether either combination would move the model past the plain 2d ceiling (0.9793 AUC) is unresolved.

- log_snr and log_rms are single scalars per window, averaged over Z, N, and E. Per-component auxiliary inputs (six scalars rather than two) were not tested and might retain information the averaging step discards, at the cost of tripling the auxiliary input’s dimensionality relative to what is otherwise an inexpensive correction.

- 60 s windows were not re-tested with any of the additions described in this section. The geometric argument in Section 2.3 for why short windows are more difficult applies specifically to the RAM branch, not to the raw-waveform or auxiliary branches, so the balance between branches may shift at longer window lengths.

- Gated fusion’s mixed result (10.5.5) has not been diagnosed further; the three plausible explanations offered are not distinguished by the current evidence and should be treated as open questions rather than findings.

## Appendix A. Reproduction Instructions: Dual-Channel and Auxiliary Pipeline

# Dual-channel dataset (paper's raw-waveform seq + RAM image), 6 s, maximum size
seismic-cli generate-dual-dataset \
    --eq-dir data/batched_waveforms/window_post_6s_anchored \
    --noise-dir data/batched_noise_waveforms/noise_pre_3h \
    --output-dir dataset_dual_6s --window-seconds 6 --max

# Same, with a spectrogram in place of a RAM image as the 2D channel
seismic-cli generate-spec-dual-dataset \
    --eq-dir data/batched_waveforms/window_post_6s_anchored \
    --noise-dir data/batched_noise_waveforms/noise_pre_3h \
    --output-dir dataset_specdual_6s --window-seconds 6 --max

# Plain RAM classifier with the amplitude auxiliary input (no LSTM branch)
seismic-cli generate-ram-aux-dataset \
    --eq-dir data/batched_waveforms/window_post_6s_anchored \
    --noise-dir data/batched_noise_waveforms/noise_pre_3h \
    --output-dir dataset_ramaux_6s --window-seconds 6 --max

# Dual-channel model with the amplitude auxiliary input, all three inputs present
seismic-cli generate-dual-aux-dataset \
    --eq-dir data/batched_waveforms/window_post_6s_anchored \
    --noise-dir data/batched_noise_waveforms/noise_pre_3h \
    --output-dir dataset_dualaux_6s --window-seconds 6 --max

# Spectrogram-dual model with the amplitude auxiliary input (Section 10.5.7)
seismic-cli generate-spec-dual-aux-dataset \
    --eq-dir data/batched_waveforms/window_post_6s_anchored \
    --noise-dir data/batched_noise_waveforms/noise_pre_3h \
    --output-dir dataset_specdualaux_6s --window-seconds 6 --max

cd ../cnn_earthquake/src

python cnn_lstm_classify.py --dataset-dir ../../data_downloader/dataset_dual_6s \
    --channels all --batch-size 32                # or --channels 1d / 2d
python cnn_lstm_classify.py --dataset-dir ../../data_downloader/dataset_dual_6s \
    --channels all --fusion gate --batch-size 32   # gated fusion (Section 10.5.5)
python cnn_lstm_classify.py --dataset-dir ../../data_downloader/dataset_specdual_6s \
    --channels all --fusion gate --batch-size 32   # gated fusion (Section 10.5.5)

python cnn_ram_aux.py --dataset-dir ../../data_downloader/dataset_ramaux_6s
python cnn_ram_aux.py --dataset-dir ../../data_downloader/dataset_ramaux_6s --no-aux
python cnn_ram_aux.py --dataset-dir ../../data_downloader/dataset_ramaux_6s --lr 3e-4   # hyperparameter sweep (Section 10.5.6)

python cnn_lstm_classify_aux.py --dataset-dir ../../data_downloader/dataset_dualaux_6s \
    --channels all --batch-size 32                 # or 1d / 2d / aux / 1d+aux / 2d+aux
python cnn_lstm_classify_aux.py --dataset-dir ../../data_downloader/dataset_dualaux_6s \
    --channels all --fusion gate --batch-size 32    # gated fusion (Section 10.5.5)
python cnn_lstm_classify_aux.py --dataset-dir ../../data_downloader/dataset_specdualaux_6s \
    --channels all --batch-size 32                 # spectrogram + aux (Section 10.5.7)
python cnn_lstm_classify_aux.py --dataset-dir ../../data_downloader/dataset_specdualaux_6s \
    --channels 2d+aux --batch-size 32               # spectrogram + aux, no LSTM (Section 10.5.7)

# Late-fusion stacking, given two already-trained --channels 1d / --channels 2d checkpoints
python cnn_lstm_stack.py --dataset-dir ../../data_downloader/dataset_specdual_6s \
    --ckpt-1d trained_model_cnnlstm_classify_1d/best_cnnlstm_classify.pth \
    --ckpt-2d trained_model_cnnlstm_classify_2d/best_cnnlstm_classify.pth

## Appendix B. Reproduction Instructions: Original RAM + CNN-Only Pipeline

# 1. Arrival-anchored 6 s windows from downloaded 60 s data
seismic-cli anchor-windows \
    --source-dir data/batched_waveforms/window_post_60s \
    --output-base-dir data/batched_waveforms -t 6

# 2. Maximum-size balanced, station-disjoint 6 s dataset
seismic-cli generate-dataset \
    --eq-dir data/batched_waveforms/window_post_6s_anchored \
    --noise-dir data/batched_noise_waveforms \
    --output-dir dataset_6s_max \
    --window-seconds 6 --overlap 0.5 --max --max-windows-per-station 20

# 3. Train (short preset auto-selected by --window-seconds)
python src/model/cnn_train.py --dataset-dir dataset_6s_max \
    --save-dir trained_model_6s --window-seconds 6

# 4. STA/LTA baseline on the identical test windows (parameters auto-derived)
seismic-cli eval-sta-lta \
    --manifest-path dataset_6s_max/manifest.csv \
    --split test --window-seconds 6 --overlap 0.5

Full option reference: data_downloader/README.md.